THE KILN CAFE

Risk Assessment & Method Statement

This document is a template. It contains the standard hazards and controls that apply to a pottery painting studio in the UK. Sections marked in grey need completing with details specific to The Kiln Cafe.

It must be reviewed and adopted by a competent person. Review at least annually, and after any incident or significant change.

# 1. Studio details

Studio name  _________________________________________________

Address  _____________________________________________________

Responsible person  __________________________________________

Date of assessment  __________________________________________

Review date  _________________________________________________

Assessment carried out by  ___________________________________

# 2. Legal framework

This assessment is made under:

- Health and Safety at Work etc. Act 1974 — general duty of care to employees and visitors

- Management of Health and Safety at Work Regulations 1999 — requirement to assess risks

- Control of Substances Hazardous to Health Regulations 2002 (COSHH) — glazes, cleaning chemicals

- Provision and Use of Work Equipment Regulations 1998 (PUWER) — kiln, wheel, tools

- Manual Handling Operations Regulations 1992 — lifting kiln shelves, boxes of bisque

- Regulatory Reform (Fire Safety) Order 2005 — fire risk assessment, means of escape

- RIDDOR 2013 — reporting of injuries, diseases and dangerous occurrences

- Workplace (Health, Safety and Welfare) Regulations 1992 — lighting, ventilation, temperature, welfare

- Electricity at Work Regulations 1989 — kiln supply, PAT testing

# 3. Persons at risk

- Staff — full and part time

- Customers, including children

- Visitors, contractors, delivery drivers

- Anyone with a disability or specific access need

- Expectant mothers (see Section 8)

# 4. Hazard assessment

Risk rating: Likelihood (1–5) × Severity (1–5) = Risk. 1–4 low, 5–9 medium, 10–25 high.

## 4.1 Kiln — heat and burns

| Hazard | Who / how harmed | Controls in place |
| --- | --- | --- |
| Hot kiln surfaces | Staff — burns from contact during or after firing | Kiln in restricted area. Heat-resistant gloves provided. Kiln not opened until below 50°C. Warning signage. |
| Kiln room heat | Staff — heat stress, dehydration | Ventilation to outside. Door kept closed during firing. Staff not to work extended periods in kiln room during firing. |
| Kiln fumes | Staff — respiratory irritation from firing gases | Mechanical extraction fitted and maintained. Kiln fired outside opening hours where practicable. Room ventilated. |
| Electrical | Staff — electric shock | Kiln on dedicated circuit. Annual inspection by competent electrician. PAT testing. No access to terminals. |

Kiln make / model  ___________________________________________

Last electrical inspection  __________________________________

Extraction system serviced  __________________________________

## 4.2 Glazes and pigments — COSHH

All glazes and pigments used must have a current Safety Data Sheet (SDS) on file. Studios must assume some glazes contain hazardous substances until proven otherwise.

| Hazard | Who / how harmed | Controls in place |
| --- | --- | --- |
| Glaze ingestion | Customers, especially children — hand to mouth contact | Only non-toxic, food-safe certified glazes used for customer painting. No lead-bearing glazes on premises. Handwashing before eating. Staff supervision. |
| Skin contact | Staff, customers — irritation, dermatitis | Aprons provided. Handwashing facilities with soap. Staff who handle glaze regularly to use gloves. |
| Dust inhalation | Staff — silica dust from dry glaze, bisque dust | Wet cleaning only — no dry sweeping. Dry glaze powder mixed in ventilated area with RPE. Spills cleaned wet immediately. |
| Eye contact | Staff, customers — irritation from splash | Eye wash available. Staff know location. Splash risk low at customer painting. |

Glaze supplier  ______________________________________________

SDS folder location  _________________________________________

COSHH assessment completed by  _______________________________

## 4.3 Manual handling

| Hazard | Who / how harmed | Controls in place |
| --- | --- | --- |
| Kiln shelves | Staff — back injury, crush injury from dropping | Two-person lift for large shelves. Shelves stored at waist height. Training in safe lifting given. |
| Bisque deliveries | Staff — back injury from repeated lifting | Trolley available. Boxes broken down before moving. Delivery placed close to storage. |
| Glaze buckets | Staff — back injury, spill | Buckets not filled beyond 10kg. Decanted at low level. |

## 4.4 Slips, trips and falls

| Hazard | Who / how harmed | Controls in place |
| --- | --- | --- |
| Wet floors | Everyone — slip injury | Spills cleaned immediately. Wet floor signs used. Non-slip flooring in wet areas. |
| Trailing cables | Everyone — trip injury | Cables routed away from walkways or covered. No extension leads across floors. |
| Clutter | Everyone — trip injury, blocked escape route | Walkways and fire exits kept clear. Daily opening check includes clear routes. |

## 4.5 Fire

A separate Fire Risk Assessment is required under the Regulatory Reform (Fire Safety) Order 2005. This section summarises only.

- Fire extinguishers: located, serviced annually, staff know locations

- Fire exits: signed, unobstructed, unlocked during opening hours

- Kiln: not left unattended overnight unless fitted with automatic cut-off and firing outside occupied hours

- Fire alarm tested weekly, log kept

- Evacuation drill at least annually

Fire Risk Assessment date  ___________________________________

Extinguisher service date  ___________________________________

Assembly point  ______________________________________________

Fire warden(s)  ______________________________________________

## 4.6 Children

| Hazard | Who / how harmed | Controls in place |
| --- | --- | --- |
| Access to kiln area | Children — burns | Kiln area not accessible to customers. Door kept closed. Staff-only signage. |
| Glaze ingestion | Children — ingestion | Non-toxic glazes only in customer areas. Parental supervision required — stated at booking. |
| Breakages | Children — cuts from broken bisque | Staff clear breakages immediately. Children not to carry pieces unsupervised. |

# 5. Method statement — daily operations

## 5.1 Opening

- Check fire exits clear and unlocked

- Check walkways clear

- Visual check of kiln area — no overnight issues

- Check first aid kit stocked

- Check handwashing facilities — soap, towels

- Check glaze station — lids on, no spills

- Log any issues in the day book

## 5.2 During service

- Aprons offered to all customers

- Spills cleaned immediately, wet method

- Glaze pots returned to station after use

- No food or drink at painting tables where glaze in use

- Handwashing reminded before customers eat

## 5.3 Kiln loading

- Kiln confirmed cold — below 50°C — before opening

- Heat-resistant gloves worn if any doubt

- Two-person lift for large shelves

- Load logged — what, when, programme

- Kiln room ventilation on before firing starts

- Firing programme confirmed before start

## 5.4 Kiln unloading

- Kiln confirmed cold before opening

- Pieces removed carefully — sharp edges possible

- Breakages cleared immediately, wet method

- Shelves inspected for damage

## 5.5 Closing

- All glaze lids on

- Wet clean of tables and floor

- Waste disposed of correctly — no glaze down the drain

- Kiln programme confirmed if firing overnight

- Fire exits clear

- Electrical items off except kiln if firing

# 6. First aid

First aider(s)  ______________________________________________

First aid kit location  ______________________________________

Accident book location  ______________________________________

All accidents must be recorded. Reportable incidents under RIDDOR must be reported to the HSE within the required timeframe:

- Death or specified injury — report immediately, then within 10 days

- Over-7-day incapacitation — within 15 days

- Occupational disease — as soon as diagnosed

- Dangerous occurrence — immediately

Report online at www.hse.gov.uk/riddor or by phone for fatal/major incidents.

# 7. Training

All staff must complete induction training before working unsupervised. Record kept in the app under Training.

- Fire procedure and assembly point

- Kiln safety — never open hot, gloves, extraction

- COSHH — glazes, SDS location, spill procedure

- Manual handling — kiln shelves, deliveries

- First aid — who, where the kit is, accident book

- Cleaning — wet method only, no dry sweeping

- Children — supervision expectations, kiln area restriction

Training records held by  ____________________________________

# 8. Specific groups

## Expectant mothers

A specific risk assessment must be completed when a member of staff notifies the studio of pregnancy. Consider: manual handling limits, heat exposure near the kiln, chemical exposure, rest breaks, standing time.

## Young persons (under 18)

A specific assessment is required. Consider: inexperience, kiln access, manual handling limits, supervision level.

## Lone working

If staff work alone, a lone working procedure is required. Consider: kiln firing while alone, first aid, means of raising alarm, check-in arrangements.

Lone working procedure in place? Y / N  ______________________

# 9. Review

| Date | Reviewed by | Changes made | Next review |
| --- | --- | --- | --- |

This template covers the standard hazards for a pottery painting studio under UK law. It is not legal advice. It must be reviewed and adopted by a competent person familiar with your specific premises, equipment and operations.

Generated for The Kiln Cafe · 17/07/2026